# Short User’s Guide for FTW PLC Simulator 
(version 1.4)

# G. Marrocco, D. Statovci (giuseppe.marrocco, driton.statovci)@ftw.at

# Forschungszentrum Telekommunikation Wien, Austria.

# April, 2012

# Objective

This document gives a short user’s guide for the FTW PLC simulator (PLC: power line communications), which is used to characterize PLC channels and it is implemented in MATLAB ©. The simulator is based on the FTW xDSL simulator (online available: http://www.xdsl.ftw.at/xdslsimu/) and to the following papers:

[F. Cañete, J. Cortés, L. Díez, and J. Entrambasaguas, “A channel model proposal for indoor power line communications,” in IEEE Communications Magazine, January 2011]

[T. Banwell and S. Galli, “On the symmetry of the power line channel,” in IEEE International Symposium on Power Line Communications and Its Applications, ISPLC 2001, 2001].

[Esmailian2003]: T. Esmailian, F.R. Kschischang, and P.G. Gulak, "An In-building Power Line Channel Simulator", Proceedings of the 2002 International Symposium on Power Line Communications and its Applications, ISPLC2002, pp. 31-35, March 2002, Athens, Greece.

Copyright ©: 2011-2015 by Forschungszentrum Telekommunikation Wien, Austria; All rights reserved.

# Simulator structure

This software is developed to simulate broadband Indoor power line channels in a frequency-domain. To help the user, a lot of comments have been included inside the MATLAB © source code. Furthermore, this user’s guide contains a detailed description of the simulator.

The simulator can be used in a fixed topology generation mode or in a random topology generation mode. To start an experimental evaluation is sufficient to run the script FTW_PLC_Simulator.m by selecting the operating mode, fixed or random. It is also possible to specify the number of iterations for the random operating mode by using numIter variable. The “Initial Settings” have been specified as described in Section 2.2. The topology description will be shown in MATLAB © Command Window only for the fixed operating mode. PLC channels transfer functions between the transmitter and receiver will be plotted in a Figure for the specified network topology. It is possible to avoid the Figure generation by setting to zero the figureOn variable.

The network topology, setting parameters, service routines, and experiment evaluation routine will be described in the following sections.

## Network topology description

The connection between the energy provider and the house is done trough the Service Panel (SP). Several Derivation Boxes (DBs) are present in the topology and each of these is directly connected to the SP (future extensions of the simulator could also consider DBs concatenated in bus). Each power line that starts from the DB terminates into an Outlet (OL). There are 2 different types of connections between the Outlets (OLs) and a DB: a STAR Connection (SC) and a BUS Connection (BC). In a SC each outlet is directly connected to a DB as illustrated in Figure 1, left plot (Room A).  In a BC the outlets are connected in parallel to a DB as illustrated in Figure 1, right plot (Room B).

Figure 1: Illustration of outlets connection types: Star connection and bus connection.

An OL can be terminated by impedance, which can have different values (as specified in the function PLC_impedences.m) or it can also be an open circuit (OC).

Figure 2. An example of PLC network topology.

An example of a PLC network topology is illustrated in Figure 2. It contains 5 derivation boxes, which are directly connected to the Service Panel. Two different types of connections (star and bus) are used to connect the outlets to DBs.

## Setting parameters

Depending on the operating mode (fixed or random), the simulator parameters are defined as in the script Init_Settings.m according to the following description.

For the random mode (mode=0), since the topologies are randomly generated, it is possible to specify ranges for the interested variables, thus:

- Max_numDB: represents the maximum number of DBs we want to have in the topology;

- Max_numOL: represents the maximum number of OLs we want to consider for each DB;

- minL and MaxL: represent the minimum and the maximum lengths in [m] for the cable section between the Service Panel and each DBs.

- DB_cableType: shows cable type used to connect the Service Panel and DBs (cable types are defined in function PLC_cables.m);

- OL_cableType: contains cable type which connect the DB and each outlets connected to it (cable types are defined in function PLC_cables.m)

- minOLdist and maxOLdist: represent the minimum and the maximum distance between one OL and its related DB;

Those are the only parameters that can be modified by the user in the random mode.

For the fixed mode (mode=1) the topology is unique and well defined, thus:

- DB_num: defines how many DBs we want to consider in the network topology;

- DB_cableLenghts: defines the cable section lengths in [m] between SP and DBs;

- DB_cablesType: shows cable types used to connect the Service Panel and DBs (cable types are defined in function PLC_cables.m);

- OL_list: defines how many OLs are connected to each DB;

- minOLdist: defines the minimum distance in [m] between one OL and DB;

- maxOLdist: defines the maximum distance in [m] between one OL and  DB;

- OL_cablesType: contains cable types which connect the DB and each outlets connected to it (cable types are defined in function PLC_cables.m);

- OL_conn: indicates the connection types between DBs and OLs (star ‘S’ or bus ‘B’);

- TxPos: specifies the PLC Transmitter (TX) node (the outlet to which the PLC Transmitter is connected). It is defined by two numbers which identify the DB identification (DBid) and the OL identification (OLid);

- RxPos: As for the TxPos, RxPos indicates the PLC Receiver (RX) node.

- Impedances: Structure that contains the impedances types connected to OLs.  If an impedance termination is not specified in the Impedances vector then it will automatically be recognized as an open circuit (OC). The impedances types are defined in function PLC_impedances.m.

## Service Routines

The experiment is set up by the function PLC_setup.m, in which some basic simulation parameters are specified by calling the following 3 functions;

- PLCsetupParameters: contains frequency parameters as the maximum and  minimum frequencies and frequency granularity;

- PLC_cables: contains cable models and cable parameters used in the simulator;

- PLC_impedances: specifies impedance models and contains a list of fixed impedance values. In this function are also called three other functions: RLC_PLCimp1.m, RLC_PLCimp2.m, RLC_PLCimp3.m, which are used to compute the frequency-selective impedances. The frequency-selective impedances are modeled as a parallel RLC resonant circuit.

## Experiment Evaluation

In this part, the simulator starts with the calcPLCchannels.m function which is used to define the Channel Transfer Function (CTF) matrix (H), the Insertion Loss matrix (IL), the loop transfer function (Hloop), and the Input impedance (Zin). The magnitude of CTF, IL and Hloop is given in dB, while the Zin is given in Ohm. Functions which are iteratively called inside the calcPLCchannels.m function are:

- getABCDforPLC: gets the ABCD parameters (PHI matrix) between transmitter and receiver nodes in the PLC network topology;

- getPHI: gets the ABCD parameters for a single power line section (loop);

- getTwoPortModel: gets the two port model [Z0,gamma] for a specific cable;

Other functions not here specified are the same as those used in the FTW xDSL simulator.

## Example of experimental result

Considering a reference topology as shown in Figure 3, an example of PLC channel realizations obtainable by using the FTW PLC simulator are shown in Figure 4.

Figure 3. PLC network reference topology.

Figure 4. Channel realization from reference topology.

Also the input impedance has been computed by using equation:

Zin = (Zload.*phi{1} + phi{2})./(Zload.*phi{3} + phi{4});

where  phi is the 2x2 ABCD  matrix representative of the channel, and Zload is the load impedance. The result is shown in Figure 5.

Figure 5. Input impedance of the channel.

## Simulator comparison

Here a comparison between our simulator and the Cañete simulator, online available on http://www.plc.uma.es/channels.htm , has been carried out.

Using the simple PLC network topology shown in Figure 6, we compare the results of the two simulators.

Figure 6. Cañete-based PLC network reference topology.

As described in Figure 6, for FTW PLC simulator the Cañete network topology can be represented with 3 derivation boxes, with the second one located in the Service Panel (SP). Three outlets impedances are considered Z1, Z2, Z3. In the first case (see Figure 7) these impedances are set to 1000 Ω to simulate approximately constant impedances values; for the second case (see Figure 8) Z1, Z2, Z3 impedances refer to the frequency-selective impedances RLC1, RLC2, RLC3 described in Section 2.3. The load and the generator impedances are equal to 100 Ω. Figures 7 and 8 show the comparison results.

Figure 7. Channel realization for approximately constant (1000 Ω) impedances.

Figure 8. Channel realization for frequency-selective impedances.

## Appendix A

As in [Golden] the Insertion Loss has been introduced for a measurement perspective, because it is difficult to access the source voltage Vgen due to internal loading of the generator Zgen.

Figure 9. An example of PLC network topology.

The Channel Transfer Functions (CTF) is defined as the voltage ratio between source voltage and load voltage (see Figure 9):

H=VLVgen

While the Insertion Loss (IL) is defined in dB as the ratio of power across the load Zload before and after insertion of the line:

IL=10 log10Pbefore insPafter ins=20 log10VL1VL2

Where VL1 is the voltage across the load before insertion of the line, while VL2 is the voltage across the load after insertion of the line.

We can also consider the loop transfer function (Hloop) as the voltage ratio between the output and the input voltage of the loop, by considering the following:

Hloop=VLV1

By using the ABCD matrix notation the quantities mentioned above can be defined as follow:

H=20 log10ZloadA*Zload+B+Zgen*C*Zload+D

IL=20 log10Zload+ZgenA*Zload+B+Zgen*C*Zload+D

Hloop=20 log10Zload*C+DZload*A*C+B*C+A*D+B*DZload

To be consistent with the reference, the CTF, the IL and the Hloop will be given (in dB) as output of the calcPLCchannels.m function.

## Appendix B

In this section we will give some graphical descriptions of how the calcPLCchannels.m function works. Here, graphically we will show how the final ABCD matrix (PHI) is built, which represents the channel between transmitter and receiver in a PLC network topology. Two cases are analyzed:

- Case 1): TX and RX are connected to the same derivation box (RxPos{1,1}=TxPos{1,1});

- Case 2): TX and RX are connected to different derivation boxes (RxPos{1,1}≠TxPos{1,1}).

### Case 1: RxPos{1,1} = TxPos{1,1}

Figure 10 shows the PLC network topology with 3 derivation boxes where both the transmitter and the receiver are connected to the DB1.

Figure 10. PLC network topology, RxPos{1,1} = TxPos{1,1}.

The simulator analyzes the DBs and for each of them computes the ABCD matrix for the loop between SP and DBs (see Figure 11):

Figure 11. PLC network topology, new matrices: phi_SPbox{DBs}.

For each loop connected to each DBs the total ABCD matrix is computed considering the termination impedances connected to the outlets:

Code:

For each DBs and for each Zterm

Zinbt = (tmp_phi{1}.*Zterm + tmp_phi{2})./(tmp_phi{3}.*Zterm + tmp_phi{4});

phi_imp = {1, 0, 1./Zinbt, 1};

phi_tot{DBs,1} = ABCDprod(phi_tot{DBs,1},phi_imp);

Figure 12. PLC network topology, new matrices: phi_tot{DBs,1}, phi_lineGen, phi_load.

As shown in Figure 12, the ABCD matrices for the loops between source and DB (phi_lineGen) and between DB1 and the load (phi_load) are also computed.

For the DBs on which no TX and RX are connected, phi_SPbox and phi_tot matrices are multiplied together (see Figure 13):

Code:

For each derivation boxes

if (DBs ~= TxPos{1,1})&&(DBs ~= RxPos{1,1}),

phi_tot{DBs,1} = ABCDprod(phi_tot{DBs,1},phi_SPbox{DBs,1});

end

Figure 13. PLC network topology, new matrices: phi_tot{DBs,1}.

In the next step, the matrix phi_tmp contains the phi_tot matrices of all the DBs on which no TX and RX are connected (see Figure 14):

Code:

for DBs=1:DB_num,       % For all the others DBs,

if DBs ~= TxPos{1,1}, % but not the ones where it is located the Tx

phi_tmp = ABCDprod(phi_tmp,phi_tot{DBs,1});

end

end

Figure 14. PLC network topology, new matrices: phi_tmp.

Now it is possible to multiply the phi_SPbox{1} matrix of the DB 1 with the phi_tmp matrix, the results are shown in Figure 15.

Code:

phi_tmp = ABCDprod(phi_tmp,phi_SPbox{TxPos{1,1},1});

Figure 15. PLC network topology, new matrices: phi_tmp.

Each ABCD matrix is an element of the PHImatrix matrix. These elements will be multiplied to obtain the final PHI matrix, which represents the final ABCD matrix representation of the channel.

Code:

PHI = {1,0,0,1};

for index=1:length(PHImatrix),

PHI =  ABCDprod(PHI,PHImatrix{1,index});

end

From “phi (= PHI)” the Insertion Loss (IL), the Channel transfer function (CTF), the loop transfer function Hloop and the Input impedance (Zin) will be computed.

Code:

A = phi{1}; B = phi{2}; C = phi{3}; D = phi{4};

%

InsLoss = (Zgen + Zload)./( (A.*Zload) + B + (C.*Zgen.*Zload) +

(D.*Zgen));

IL=20*log10(abs(InsLoss));

%

H = 20.*log10( abs( Zload ./ ( (A.*Zload)+ B + (C.*Zgen.*Zload) +

(D.*Zgen) ) ) );

%

Hloop = 20.*log10(abs( (Zload.*C + D) ./ ( (Zload.*A.*C) + B.*C + A.*D +

( (B.*D)./Zload ) ) ) );

%

Zin = ((Zload.*A) + B) ./ ((Zload.*C) + D);

%

### Case 2: RxPos{1,1} ≠ TxPos{1,1}

Figure 16 shows the PLC network topology with 4 derivation boxes and transmitter and receiver connected to different derivation boxes.

Figure 16. PLC network topology, RxPos{1,1} ≠ TxPos{1,1}.

The simulator analyzes the DBs and for each of them computes the ABCD matrix for the loop between SP and DBs (see Figure 17):

Figure 13. PLC network topology, new matrices: phi_SPbox{DBs}.

For each loop connected to each DBs the total ABCD matrix is computed considering the termination impedance connected to the outlets (see Figure 17):

Code:

For each DBs and for each Zterm

Zinbt = (tmp_phi{1}.*Zterm + tmp_phi{2})./(tmp_phi{3}.*Zterm + tmp_phi{4});

phi_imp = {1, 0, 1./Zinbt, 1};

phi_tot{DBs,1} = ABCDprod(phi_tot{DBs,1},phi_imp);

Figure 18. PLC network topology, new matrices: phi_tot{DBs,1}, phi_lineGen, phi_load.

As shown in Figure 18, the ABCD matrices for the loops between source and DB (phi_lineGen) and between DB2 and the load (phi_load) are also computed.

For the DBs on which no TX and RX are connected, phi_SPbox and phi_tot matrices are multiplied together (see Figure 19):

Code:

For each derivation boxes

if (DBs ~= TxPos{1,1})&&(DBs ~= RxPos{1,1}),

phi_tot{DBs,1} = ABCDprod(phi_tot{DBs,1},phi_SPbox{DBs,1});

end

Figure 19. PLC network topology, new matrices: phi_tot{DBs,1}.

In the next step, the matrix phi_tmp contains the phi_tot matrices of all the DBs on which no TX and RX are connected (see Figure 20):

Code:

for DBs=1:DB_num,       % For all the others DBs,

if DBs ~= TxPos{1,1}, % but not the ones where it is located the Tx

phi_tmp = ABCDprod(phi_tmp,phi_tot{DBs,1});

end

end

Figure 20. PLC network topology, new matrices: phi_tmp.

Each ABCD matrix is an element of the PHImatrix matrix. These elements will be multiplied to obtain the final PHI matrix, which represents the final ABCD matrix representation of the channel.

Code:

PHI = {1,0,0,1};

for index=1:length(PHImatrix),

PHI =  ABCDprod(PHI,PHImatrix{1,index});

end

From “phi (= PHI)” the Insertion Loss (IL), the Channel transfer function (CTF), the loop transfer function Hloop and the Input impedance (Zin) will be computed.

Code:

A = phi{1}; B = phi{2}; C = phi{3}; D = phi{4};

%

InsLoss = (Zgen + Zload)./( (A.*Zload) + B + (C.*Zgen.*Zload) +

(D.*Zgen));

IL=20*log10(abs(InsLoss));

%

H = 20.*log10( abs( Zload ./ ( (A.*Zload)+ B + (C.*Zgen.*Zload) +

(D.*Zgen) ) ) );

%

Hloop = 20.*log10(abs( (Zload.*C + D) ./ ( (Zload.*A.*C) + B.*C + A.*D +

( (B.*D)./Zload ) ) ) );

%

Zin = ((Zload.*A) + B) ./ ((Zload.*C) + D);

%

The difference between CTF, IL and Hloopwill be described in the Appendix A.

P. Golden, H. Dedieu, and K. S. Jacobsen, Fundamentals of DSL Technology, 1st ed. Auerbach Publications, July 2004.