| Adobe brick building |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EXTERNAL WALL |  |  |  |  |  |  |  |  |  |  |
| Layer |  | Material | d [m] | λ [W/m.K] |  |  | R [m2.K/W] |  |  |  |
| 1 |  | Clay plaster (0,02m) | 0,02 | 0,87 |  |  | 0,02 |  |  |  |
| 2 |  | Adobe bricks(0,25m) | 0,25 | 0,69 |  |  | 0,36 |  |  |  |
| 3 |  | Straw bale (0,25m) | 0,25 | 0,052 |  |  | 4,81 |  |  |  |
| 4 |  | Clay plaster (0,02m) | 0,02 | 0,87 |  |  | 0,02 |  |  |  |
|  |  |  |  | Σ R |  |  | 5,22 |  |  |  |
|  |  |  |  | Rsi |  |  | 0,13 |  |  |  |
|  |  |  |  | Rse |  |  | 0,04 |  |  |  |
|  |  |  |  | RT |  |  | 5,39 |  |  |  |
|  |  |  |  | U |  |  | 0,19 |  |  |  |
| Table 3.1 – U-value of the external wall of the adobe brick building |  |  |  |  |  |  |  |  |  |  |
| SLAB IN CONTACT WITH SOIL |  |  |  |  |  |  |  |  |  |
| Layer | Material |  |  |  | d [m] | λ [W/m.K] |  | R [m2.K/W] |  |
| 1 | Wooden floor (10mm) |  |  |  | 0,01 | 0,18 |  | 0,06 |  |
| 2 | Polyethylene fleece (0,002m) |  |  |  | 0,00 | 0,33 |  | 0,01 |  |
| 3 | Cement screed (0,06m) |  |  |  | 0,06 | 1,50 |  | 0,04 |  |
| 4 | SBS modified asphalt strip 2x (0,004m) |  |  |  | 0,01 | 0,17 |  | 0,05 |  |
| 5 | XPS (0,14m) |  |  |  | 0,14 | 0,03 |  | 4,38 |  |
| 6 | Concrete (0,2m) |  |  |  | 0,20 | 1,70 |  | 0,12 |  |
| 7 | Compacted gravel (0.15m) |  |  |  | 0,15 | 1,50 |  | 0,10 |  |
|  |  |  |  |  |  | Σ R |  | 4,74 |  |
|  |  |  |  |  |  | Rsi |  | 0,13 |  |
|  |  |  |  |  |  | Rse |  | 0,04 |  |
|  |  |  |  |  |  | RT |  | 4,91 |  |
|  |  |  |  |  |  | U |  | 0,20 |  |
| Table 3.4 – U-value of the slab in contact wih soil of the adobe brick building |  |  |  |  |  |  |  |  |  |

| WARM ROOF |  |  |  |  |
| --- | --- | --- | --- | --- |
| Layer | Material | d [m] | λ [W/m.K] | R [m2.K/W] |
| 1 | Spanish clay tiles (0.008m) | 0,01 | 0,57 | 0,01 |
| 2 | Timber roofing lath 40×60 mm (0.04m) | 0,04 | 0,12 | 0,33 |
| 3 | Air space (min. 0.04m) | 0,04 | 0,00 | 0,18 |
| 4 | Ventia Titanium Plus 190G (Sd≈0.02 m) (0.001m) | 0,00 | 0,22 | 0,00 |
| 5 | PIR boards (Thermano Roof AL) (0.12m) | 0,12 | 0,02 | 5,22 |
| 6 | Bitumen felt (0.004m) | 0,00 | 0,19 | 0,02 |
| 7 | OSB board 2x (0.025m) | 0,03 | 0,13 | 0,19 |
| 8 | Air space | 0,00 | 0,00 | 0,18 |
| 9 | Timber rafter 200×240 mm(0.2m) | 0,20 | 0,12 | 1,67 |
| 10 | Plasterboard (0.0125m) | 0,01 | 0,25 | 0,05 |
|  |  |  | Σ R | 7,86 |
|  |  |  | Rsi | 0,13 |
|  |  |  | Rse | 0,04 |
|  |  |  | RT | 8,03 |
|  |  |  | U | 0,12 |
| Table 3.7 – U-value of the warm roof of the adobe brick building |  |  |  |  |

| INTERIOR WALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Layer | Material | d [m] | λ [W/m.K] | R [m2.K/W] |
| 1 | Lime-cement (0,01m) | 0,01 | 0,48 | 0,02 |
| 2 | Lime-sand bricks (0,115m) | 0,12 | 0,38 | 0,30 |
| 3 | Lime-cement (0,01m) | 0,01 | 0,48 | 0,02 |
|  |  |  | Σ R | 0,34 |
|  |  |  | Rsi | 0,13 |
|  |  |  | Rse | 0,04 |
|  |  |  | RT | 0,51 |
|  |  |  | U | 1,94 |
| Table 3.10 – U-value of the interior wall of the adobe brick building |  |  |  |  |

| Rammed earth building |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EXTERNAL WALL |  |  |  |  |  |  |  |  |  |
| Layer |  | Material | d [m] |  | λ [W/m.K] |  | R [m2.K/W] |  |  |
| 1 |  | Clay plaster (0.02 m) | 0,025 |  | 0,87 |  | 0,03 |  |  |
| 2 |  | Rammed earth (0.25 m) | 0,25 |  | 0,7 |  | 0,36 |  |  |
| 3 |  | Straw bale (0,25m) | 0,25 |  | 0,052 |  | 4,81 |  |  |
| 4 |  | Air space (0,03m) | 0,03 |  | 0 |  | 0,00 |  |  |
| 5 |  | Cembrit board (0.012 m) | 0,012 |  | 0,3 |  | 0,04 |  |  |
|  |  |  |  |  | Σ R |  | 5,23 |  |  |
|  |  |  |  |  | Rsi |  | 0,13 |  |  |
|  |  |  |  |  | Rse |  | 0,04 |  |  |
|  |  |  |  |  | RT |  | 5,40 |  |  |
|  |  |  |  |  | U |  | 0,19 |  |  |
| Table 3.2 – U-value of the external wall of the rammed earth building |  |  |  |  |  |  |  |  |  |
| SLAB IN CONTACT WITH SOIL |  |  |  |  |  |  |  |  |
| Layer | Material |  |  | d [m] |  | λ [W/m.K] |  | R [m2.K/W] |
| 1 | Wooden floor (10mm) |  |  | 0,01 |  | 0,18 |  | 0,06 |
| 2 | Polyethylene fleece (0,002m) |  |  | 0,00 |  | 0,33 |  | 0,01 |
| 3 | Cement screed (0,06m) |  |  | 0,06 |  | 1,50 |  | 0,04 |
| 4 | SBS modified asphalt strip 2x (0,004m) |  |  | 0,01 |  | 0,17 |  | 0,05 |
| 5 | XPS (0,14m) |  |  | 0,14 |  | 0,03 |  | 4,38 |
| 6 | Concrete (0,2m) |  |  | 0,20 |  | 1,70 |  | 0,12 |
| 7 | Compacted gravel (0.15m) |  |  | 0,15 |  | 1,50 |  | 0,10 |
|  |  |  |  |  |  | Σ R |  | 4,74 |
|  |  |  |  |  |  | Rsi |  | 0,13 |
|  |  |  |  |  |  | Rse |  | 0,04 |
|  |  |  |  |  |  | RT |  | 4,91 |
|  |  |  |  |  |  | U |  | 0,20 |
| Table 3.5 – U-value of the slab in contact wih soil of the rammed earth building |  |  |  |  |  |  |  |  |

| WARM ROOF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Layer | Material |  |  |  | d [m] | λ [W/m.K] |  | R [m2.K/W] |  |
| 1 | Spanish clay tiles (0.008m) |  |  |  | 0,01 | 0,57 |  | 0,01 |  |
| 2 | Timber roofing lath 40×60 mm (0.04m) |  |  |  | 0,04 | 0,12 |  | 0,33 |  |
| 3 | Air space (min. 0.04m) |  |  |  | 0,04 | 0,00 |  | 0,18 |  |
| 4 | Ventia Titanium Plus 190G (Sd≈0.02 m) (0.001m) |  |  |  | 0,00 | 0,22 |  | 0,00 |  |
| 5 | PIR boards (Thermano Roof AL) (0.12m) |  |  |  | 0,12 | 0,02 |  | 5,22 |  |
| 6 | Bitumen felt (0.004m) |  |  |  | 0,00 | 0,19 |  | 0,02 |  |
| 7 | OSB board 2x (0.025m) |  |  |  | 0,03 | 0,13 |  | 0,19 |  |
| 8 | Air space |  |  |  | 0,00 | 0,00 |  | 0,18 |  |
| 9 | Timber rafter 200×240 mm(0.2m) |  |  |  | 0,20 | 0,12 |  | 1,67 |  |
| 10 | Plasterboard (0.0125m) |  |  |  | 0,01 | 0,25 |  | 0,05 |  |
|  |  |  |  |  |  | Σ R |  | 7,86 |  |
|  |  |  |  |  |  | Rsi |  | 0,13 |  |
|  |  |  |  |  |  | Rse |  | 0,04 |  |
|  |  |  |  |  |  | RT |  | 8,03 |  |
|  |  |  |  |  |  | U |  | 0,12 |  |
| Table 3.8 – U-value of the warm roof of the rammed earth building |  |  |  |  |  |  |  |  |  |
| INTERIOR WALL |  |  |  |  |  |  |  |  |  |
| Layer |  | Material | d [m] | λ [W/m.K] |  |  | R [m2.K/W] |  |
| 1 |  | Lime-cement (0,01m) | 0,01 | 0,48 |  |  | 0,02 |  |
| 2 |  | Lime-sand bricks (0,115m) | 0,12 | 0,38 |  |  | 0,30 |  |
| 3 |  | Lime-cement (0,01m) | 0,01 | 0,48 |  |  | 0,02 |  |
|  |  |  |  | Σ R |  |  | 0,34 |  |
|  |  |  |  | Rsi |  |  | 0,13 |  |
|  |  |  |  | Rse |  |  | 0,04 |  |
|  |  |  |  | RT |  |  | 0,51 |  |
|  |  |  |  | U |  |  | 1,94 |  |
| Table 3.11 – U-value of the interior wall of the rammed earth building |  |  |  |  |  |  |  |  |

| Traditional building |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EXTERNAL WALL |  |  |  |  |  |  |  |  |  |  |  |
| Layer |  |  | Material | d [m] |  | λ [W/m.K] |  | R [m2.K/W] |  |  |  |
| 1 |  |  | Lime-cement (0,01m) | 0,01 |  | 0,48 |  | 0,02 |  |  |  |
| 2 |  |  | Lime-sand bricks (0,24m) | 0,24 |  | 0,38 |  | 0,63 |  |  |  |
| 3 |  |  | Cement based adhesive (0,005m) | 0,005 |  | 0,54 |  | 0,01 |  |  |  |
| 4 |  |  | Mineral wool (0,16m) | 0,16 |  | 0,036 |  | 4,44 |  |  |  |
| 5 |  |  | Cement mortar (0,002 m) | 0,002 |  | 0,054 |  | 0,04 |  |  |  |
| 6 |  |  | Fiberglass (0,001m) | 0,001 |  | 0,054 |  | 0,02 |  |  |  |
| 7 |  |  | Cement mortar (0,002 m) | 0,002 |  | 0,054 |  | 0,04 |  |  |  |
| 8 |  |  | Silicate primer (0.001m) | 0,001 |  | 0,8 |  | 0,00 |  |  |  |
| 9 |  |  | Silicon plaster (0,008 m) | 0,008 |  | 0,66 |  | 0,01 |  |  |  |
|  |  |  |  |  |  | Σ R |  | 5,21 |  |  |  |
|  |  |  |  |  |  | Rsi |  | 0,13 |  |  |  |
|  |  |  |  |  |  | Rse |  | 0,04 |  |  |  |
|  |  |  |  |  |  | RT |  | 5,38 |  |  |  |
|  |  |  |  |  |  | U |  | 0,19 |  |  |  |
| Table 3.3 – U-value of the external wall of the traditional building |  |  |  |  |  |  |  |  |  |  |  |
| SLAB IN CONTACT WITH SOIL |  |  |  |  |  |  |  |  |  |
| Layer | Material |  |  | d [m] |  | λ [W/m.K] |  | R [m2.K/W] |  |
| 1 | Wooden floor (10mm) |  |  | 0,01 |  | 0,18 |  | 0,06 |  |
| 2 | Polyethylene fleece (0,002m) |  |  | 0,00 |  | 0,33 |  | 0,01 |  |
| 3 | Cement screed (0,06m) |  |  | 0,06 |  | 1,50 |  | 0,04 |  |
| 4 | SBS modified asphalt strip 2x (0,004m) |  |  | 0,01 |  | 0,17 |  | 0,05 |  |
| 5 | XPS (0,14m) |  |  | 0,14 |  | 0,03 |  | 4,38 |  |
| 6 | Concrete (0,2m) |  |  | 0,20 |  | 1,70 |  | 0,12 |  |
| 7 | Compacted gravel (0.15m) |  |  | 0,15 |  | 1,50 |  | 0,10 |  |
|  |  |  |  |  |  | Σ R |  | 4,74 |  |
|  |  |  |  |  |  | Rsi |  | 0,13 |  |
|  |  |  |  |  |  | Rse |  | 0,04 |  |
|  |  |  |  |  |  | RT |  | 4,91 |  |
|  |  |  |  |  |  | U |  | 0,20 |  |
| Table 3.6 – U-value of the slab in contact wih soil of the traditional building |  |  |  |  |  |  |  |  |  |

| TRADITIONAL FLAT ROOF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Layer | Material |  |  | d [m] |  | λ [W/m.K] |  | R [m2.K/W] |  |
| 1 | Soil (0,06m) |  |  | 0,06 |  | 0,70 |  | 0,09 |  |
| 2 | Polyethylene fleece (0,003m) |  |  | 0,003 |  | 0,33 |  | 0,01 |  |
| 3 | HDPE drainage board (0,025m) |  |  | 0,025 |  | 0,04 |  | 0,63 |  |
| 4 | Polyethylene fleece (0,0036m) |  |  | 0,0036 |  | 0,33 |  | 0,01 |  |
| 5 | SBS modified asphalt strip 2x (0,008m) |  |  | 0,008 |  | 0,19 |  | 0,04 |  |
| 6 | EPS (0,2m) |  |  | 0,2 |  | 0,04 |  | 5,56 |  |
| 7 | EPS sloping (0,04m) |  |  | 0,04 |  | 0,04 |  | 1,11 |  |
| 8 | Bitumen felt (0,004m) |  |  | 0,004 |  | 0,02 |  | 0,21 |  |
| 9 | Reinforced concrete (0,20m) |  |  | 0,2 |  | 2,00 |  | 0,10 |  |
| 10 | Lime-cement (0,01m) |  |  | 0,01 |  | 0,75 |  | 0,01 |  |
|  |  |  |  |  |  | Σ R |  | 7,76 |  |
|  |  |  |  |  |  | Rsi |  | 0,13 |  |
|  |  |  |  |  |  | Rse |  | 0,04 |  |
|  |  |  |  |  |  | RT |  | 7,93 |  |
|  |  |  |  |  |  | U |  | 0,13 |  |
| Table 3.9 – U-value of the flat roof of the traditional building |  |  |  |  |  |  |  |  |  |
| INTERIOR WALL |  |  |  |  |  |  |  |  |  |
| Layer |  | Material | d [m] |  | λ [W/m.K] |  | R [m2.K/W] |  |
| 1 |  | Lime-cement (0,01m) | 0,01 |  | 0,48 |  | 0,02 |  |
| 2 |  | Lime-sand bricks (0,115m) | 0,12 |  | 0,38 |  | 0,30 |  |
| 3 |  | Lime-cement (0,01m) | 0,01 |  | 0,48 |  | 0,02 |  |
|  |  |  |  |  | Σ R |  | 0,34 |  |
|  |  |  |  |  | Rsi |  | 0,13 |  |
|  |  |  |  |  | Rse |  | 0,04 |  |
|  |  |  |  |  | RT |  | 0,51 |  |
|  |  |  |  |  | U |  | 1,94 |  |
| Table 3.12 – U-value of the interior wall of the traditional building |  |  |  |  |  |  |  |  |

Table 4.1 – A1–A3 Environmental Impact Results of Adobe, Rammed Earth, and Traditional Buildings

Figure 4.1 – Relative comparison of environmental impacts for the A4 transport stage across Adobe, Rammed Earth, and Traditional construction systems.

Figure 4.2 Relative comparison of environmental impacts for the A5 construction and installation stage across Adobe, Rammed Earth, and Traditional construction systems.

Figure 4.3 C1 Demolition - Environmental impacts of the C1 demolition stage for Adobe, Rammed Earth, and Traditional construction systems. In the C1 demolition stage, the environmental impacts are mainly related to machinery use and energy consumption during dismantling activities. The results indicate only minor differences between the three construction systems, as similar demolition assumptions and equipment are applied. However, the traditional construction system shows slightly higher impacts due to the greater mass and mechanical resistance of concrete-based elements. This stage includes demolition activities, transport of waste materials, and waste processing or recycling.

Figure 4.4 C2 Transport to waste processing - Environmental impacts of the C2 transport to waste processing stage for Adobe, Rammed Earth, and Traditional construction systems. For the C2 transport stage, differences between the construction systems become more visible. The traditional variant exhibits higher impacts, which can be attributed to larger quantities of demolition waste and longer transport requirements for mixed mineral materials. In contrast, adobe and rammed earth systems benefit from lower material mass and simpler waste streams, resulting in reduced transport-related environmental burdens.

Figure 4.5 C3 Waste processing - Environmental impacts of the C3 waste processing stage for Adobe, Rammed Earth, and Traditional construction systems. The C3 waste processing stage shows the clearest distinction between earthen and traditional construction systems. Earthen materials demonstrate lower environmental impacts due to their ability to be reused, recycled as mineral material, or returned to the natural environment with minimal processing. Traditional construction systems, on the other hand, require more energy-intensive waste treatment processes, particularly for concrete and fired brick materials.

Figure 5.8 – Comparison of operational environmental impacts (B6–B7) for Adobe, Rammed Earth, and Traditional residential buildings across selected impact categories.

Table 5.1 – Life-cycle environmental impact results for the representative rammed earth residential building. The results indicate that the operational stage (B6–B7) represents the dominant contribution to total impacts, while the product stage (A1–A3) remains the main source of embodied environmental burdens.