Sample text 2

We discuss the modeling of the magnetic fields that surround non-ferromagnetic materials. In relevant applications, the reaction field formed by inserting the examined sample in a homogeneous field facilitates the calculation of the magnetic susceptibility. The paper describes the procedure to verify the validity of the general relationship for calculating the susceptibility from the reaction field for samples of various shapes and materials. The actual modeling utilizes software based on the finite element method (FEM). Samples of paramagnetic and diamagnetic materials, such as aluminum and copper, are are placed in a homogeneous field within an air filled block; the samples are spherical, cylindrical, and cuboidal.

Magnetic susceptibility is a physical quantity describing material properties in an external magnetic field. The quantity is defined as the ratio between the magnetization M of a material in a magnetic field and the field intensity H:

| M = χm H. | (1) |
| --- | --- |

All materials can be classified into three groups by the magnetic susceptibility value. The corresponding sets are as follows:

- diamagnetic,	− 1 < χm < 0;

- paramagnetic,	0 < χm << 1;

- ferromagnetic,	χm >> 1.

Magnetic susceptibility measurement is typically performed via diverse methods, such as Faraday´s scale, Guoy‘s scale, or the inductive technique using a SQUID magnetometer. A detailed description of these approaches is proposed in several related sources. The discussed task can be also carried out by means of magnetic resonance (MR), which nevertheless yields only approximate results, requiring one of the above-mentioned methods to verify the outcome.

The present paper analyzes the magnetic susceptibility of materials that appear to be non-ferromagnetic from the macroscopic perspective. The individual chapters then characterize the modeling of the reaction field in the vicinity of variously shaped samples (spherical, cylindrical, and cuboidal) of different materials (aluminium, copper).

2. Methods

MR-based susceptibility measurement exploits the assumption of a constant magnetic flux in the working space of a superconducting magnet. Inserting a specimen with magnetic susceptibility s causes local deformation of a previously homogeneous magnetic field (Fig. 1).

The magnitude of such a deformation depends on the difference between the magnetic susceptibilities of the sample (χs) and its vicinity (v); the volume and shape of the sample; and the magnitude of the basic field B0.

Let there be a static magnetic field described by B0 in the direction of axis z. Assume that a cylindrical specimen of diameter d and length l (ls>>d) is inserted into the magnetic field, parallel to the direction of B0. The behaviour of the magnetic flux density Bz(x) in position y = 0 and z = 0 on a straight line is shown in Figure 1. The difference between a change in the magnetic field in the vicinity of the specimen and the value of the static magnetic field B0 is referred to as the reaction field B.

As can be seen, the specimen affects not only the actual volume but also the vicinity of the field. Then, according to [1], the magnetic flux density inside the specimen will be equal to

| . | (2) |
| --- | --- |

Assume a constant magnetic flux  through the normal area of the cross-section Sz of the working space of the given magnet. We have

| , | (3) |
| --- | --- |

from which it is evident that the magnetic flux density outside the specimen is changed, resulting in a shape that can be considered the superposition of the homogeneous field B0 and the reaction field B.

From Eqs. 2 and 3, we derived the following formula to calculate the magnetic susceptibility from a 3D map of the reaction field being measured [1]:

| , | (4) |
| --- | --- |

where Bv is the reaction field in the vicinity of the specimen, Vs is the volume of the specimen, and B0 is the static magnetic field.

2. Experiment

The space configuration of the sample is represented in Fig. 1. The block space (50x50x100 mm) filled with air has the magnetic susceptibility of χA = 0, denoted by number 1 in Fig. 2. The center of the space holds spherical, cylindrical, and cuboidal samples of copper and aluminium; these materials exhibit different magnetic susceptibilities χS. The individual dimensions and parameters are summarized in Tab. 1.

Tab.1: Parameters of the samples

| Shape | Sphere | Cylinder | Block |
| --- | --- | --- | --- |
| Length | Diameter: 5 mm | 50 mm | 50 mm |
| Cross-section |  | Diameter: 5 mm | 5 mm x 5 mm |

We used a paramagnetic and a diamagnetic sample, or aluminum (χmAl = 2.2∙10-5) and copper (χmCu = -9.2∙10-6), respectively. The cross-section plane, labeled with number 3 in Fig. 2, is located in the center of the cubic space.

The model was designed using the finite element method (FEM), with the boundary conditions chosen according to the formula for the magnetic potential in order to yield the value of B = 4.7 T in the homogenous static magnetic field. The given formula is

| . | (5) |
| --- | --- |

The analysis is based on the FEM. To describe the quasi-stable model, we use the reduced Maxwell’s equations

| rot H = J, div B = 0, | (6), (7) |
| --- | --- |

where H is the magnetic field intensity vector, B is the magnetic flux density vector, and J denotes the current density vector.

For an irrotational field, the above equation (3) assumes the form

| rot H = 0. | (8) |
| --- | --- |

The material relations are then outlined by equation

| . | (9) |
| --- | --- |

The modeled reaction field B of the samples is shown in the cross-section plane, Fig. 2. The value ΔB = Bz_emnc – 4.7 is plotted on the y axis. The related curves are indicated on the x axis in the middle of the modeled sample (Fig. 1).

To calculate the magnetic susceptibility, we employ the course of the reaction field in the x axis. The magnetic susceptibility of the modeled material is then equal to

| , | (10) |
| --- | --- |

where the sign (±) in front of the fraction depends on the material used. The + is utilized for calculating the susceptibility of paramagnetic substances, while the – enables us to obtain the susceptibility of diamagnetic ones. The values Bmax and Bmin represent the value of ΔB in the space of the sample, with Bmax denoting the field close to the inner side of the boundary of the sample and Bmin expressing the field close to the corresponding outer side. The magnetic susceptibility of the simulated samples is calculated by using the values of the reaction field ΔB from Fig. 2 and equation (7). In the table below we summarize the results of the magnetic susceptibility of all the samples.

The paper characterizes a simplified procedure and a formula to calculate magnetic susceptibility (10), both based on Eq. (4) as released in source [1]. Based on modeling the magnetic field in the vicinity of samples of non-ferromagnetic materials, we consider the formula (10) applicable to all the paramagnetic and diamagnetic material samples. The magnetic fields surrounding the samples are shown in Fig. 3.